Drone Detection Pipeline

Field Dataset Integration & Benchmark Guide

Dunakeszi Outdoor Session  ·  Vitemor Indoor Session  ·  v15 Training Pipeline

# 1  Overview

This guide documents two real-world field datasets used to test and validate the drone detection, localization, and path-tracking pipeline built with v15. The two sessions are complementary: one is an outdoor urban flight over a known GPS trajectory; the other is a controlled indoor flight at Pannónia Film Studio with marker-annotated maneuvers and high-precision telemetry.

| Property | Dunakeszi (Outdoor) | Vitemor (Indoor) |
| --- | --- | --- |
| Session ID | Dunakeszi_2025_10_25 | 250703_Vitemor_Dron |
| Date | 2025-10-20 | 2025-07-03 |
| Environment | Outdoor urban | Indoor studio |
| Drones | 1 (unknown model) | DJI Mavic Pro · Mavic 2 Pro · Mavic Mini |
| Flight phases | 1 continuous flight | 4 phases (3 scripted + 1 free-run) |
| Audio — main | 14-ch 192 kHz Brüel + 12-ch 8 kHz MEMS | 12-ch 192 kHz WAV (Grp1×6 + Grp2×6) |
| Audio — aux | — | 4-ch group3 (~21 min) |
| Ground truth | UTC flight window + GPX | 75 sample-index markers + position CSVs |
| Benchmark script | dunakeszi_benchmark.py | vitemor_indoor_benchmark.py |

# 2  Repository Layout

Recommended folder structure after cloning. All scripts auto-discover paths when --dataset-root points at the correct root.

deployment/v2/ ├── drone_detection_v15.py # Main training + inference pipeline ├── dunakeszi_audit_v2.py # Outdoor pre-processing (generates .npy caches) ├── dunakeszi_benchmark.py # Outdoor benchmark evaluator ├── vitemor_indoor_benchmark.py # Indoor benchmark evaluator ├── custom_drone_dataset_builder.py # Dataset augmentation builder ├── test_dataset_downloader.py # Held-out test data downloader │ ├── dunakeszi_data/ # Outdoor raw data (download separately) │ ├── Dunakeszi_MEMS/Audio/ # 12 × 8 kHz WAV (MEMS microphones) │ ├── Dunakeszi_BRUEL_VIDEO/ # Raw 14-ch 192 kHz Brüel WAVs │ └── Dunakeszi_DRON-ADATOK/ # GPX telemetry + flight logs │ ├── dunakeszi_audit_output/ # Generated by dunakeszi_audit_v2.py │ ├── *_flight_window.npy # Pre-extracted flight-window audio caches │ └── gpx_combined.csv # Parsed GPX trackpoints │ ├── kutatas/ # Indoor raw data (download separately) │ ├── audio_files/ # 12 × 192 kHz WAV (Grp1-1..Grp2-6) │ ├── group3_audio/ # 4 × 192 kHz WAV (shorter, one phase) │ ├── markers/ # 75-event timeline in 3 formats │ ├── position/ # mavic_pro.csv / mavic_pro2.csv / mavic mini.csv │ └── q4_features.csv # BPF + mel features for all hover segments │ └── drone_v15/ # Created by training pipeline └── models/ ├── best_detection.pth └── best_localization.pth

# 3  Dependencies

# Core pip install numpy scipy pandas matplotlib soundfile librosa pip install scikit-learn torch torchaudio pip install requests remotezip pydub # soundfile is required for the indoor benchmark (seek-based WAV loading) pip install soundfile # Optional — extends heuristic scoring quality pip install librosa # already listed above

pydub prints a ffmpeg warning on startup. This is harmless for .wav and .mp3 files. Only needed for .m4a / .ogg conversion.

# 4  Session 1 — Dunakeszi Outdoor

## 4.1  Dataset Overview

A single continuous drone flight over an outdoor urban site. The Brüel array (14 channels, 192 kHz) and a 12-microphone MEMS array (8 kHz) recorded simultaneously for ~83 minutes. The flight window is annotated precisely from the on-board drone log.

| Parameter | Value |
| --- | --- |
| Takeoff (UTC) | 2025-10-20 12:57:21 |
| Landing (UTC) | 2025-10-20 13:04:09 |
| Duration | 408 s (6.8 min) |
| Brüel SR | 192 000 Hz · 14 channels · 24-bit |
| MEMS SR | 8 000 Hz · 16-bit · mono |
| MEMS files | 12 WAV files · ~8 722 s each (~2.4 h) |
| GPX points | 9 028 total · 4 066 in flight window |
| Sync anchor | marker #1 "taps sync" at sample 1 646 731 |

## 4.2  Extracting the Flight Window — Step by Step

### Step 1  Run the Audit Script

dunakeszi_audit_v2.py connects to the Nextcloud share, downloads only the two Brüel WAVs that bracket the flight, extracts the exact flight window from each, and writes compressed .npy caches.

# 1. Set SHARE_URL at the top of dunakeszi_audit_v2.py # 2. Run — takes ~10 min on first run, skips existing files on re-run python dunakeszi_audit_v2.py

The script downloads two files selectively using HTTP Range headers:

| File | Wall-clock start | Seconds pulled | Purpose |
| --- | --- | --- | --- |
| 251020VITEMOROM1AT01I.wav | 12:50:34 UTC | 600 s | Pre-flight tail + flight start |
| 251020VITEMOROM1AT01J.wav | 12:59:28 UTC | 360 s | Main flight window |

Output caches written to dunakeszi_audit_output/:

- 251020VITEMOROM1AT01I_flight_window.npy  — float32, shape (N_frames, 14)

- 251020VITEMOROM1AT01J_flight_window.npy  — float32, shape (N_frames, 14)

- gpx_combined.csv  — parsed GPX trackpoints with UTC timestamps

### Step 2  Verify the Cache

import numpy as np arr = np.load("dunakeszi_audit_output/251020VITEMOROM1AT01J_flight_window.npy", mmap_mode="r") print(f"Shape: {arr.shape} SR: 192000 Duration: {arr.shape[0]/192000:.1f}s") # Expected: Duration ~280 s (flight end minus AT01J start)

### Step 3  Manual Flight Window Extraction (without the audit script)

If you have the raw WAV files and want to extract the flight window directly:

import soundfile as sf import numpy as np SR = 192_000 TAKEOFF_UTC = "2025-10-20T12:57:21+00:00" FILE_START_UTC = "2025-10-20T12:59:28+00:00" # AT01J from datetime import datetime, timezone file_start = datetime.fromisoformat(FILE_START_UTC) takeoff = datetime.fromisoformat(TAKEOFF_UTC) landing = datetime(2025, 10, 20, 13, 4, 9, tzinfo=timezone.utc) # Convert timestamps to sample offsets within this file s0 = max(0, int((takeoff - file_start).total_seconds() * SR)) s1 = int((landing - file_start).total_seconds() * SR) data, _ = sf.read("Dunakeszi_BRUEL_VIDEO/251020VITEMOROM1AT01J.wav", start=s0, stop=s1, dtype="float32", always_2d=True) print(f"Extracted: {data.shape} {data.shape[0]/SR:.1f}s")

## 4.3  Running the Outdoor Benchmark

### Quick start (heuristic only)

python dunakeszi_benchmark.py \ --dataset-root ./dunakeszi_data \ --source both \ --gt-strategy all \ --no-model

### Full run with CNN model

python dunakeszi_benchmark.py \ --dataset-root ./dunakeszi_data \ --model-dir ./drone_v15/models \ --source both \ --gt-strategy all

### MEMS only, flight-window GT, fixed threshold

python dunakeszi_benchmark.py \ --dataset-root ./dunakeszi_data \ --source mems \ --gt-strategy flight_window \ --threshold 0.65

### Brüel only, GPX altitude GT, channel 0

python dunakeszi_benchmark.py \ --dataset-root ./dunakeszi_data \ --source bruel \ --gt-strategy gpx_altitude \ --bruel-channel 0 \ --gpx-alt-threshold 5.0

## 4.4  CLI Reference — Outdoor Benchmark

| Flag | Default | Description |
| --- | --- | --- |
| --dataset-root | . | Root of Dunakeszi dataset — sub-dirs auto-discovered |
| --mems-dir | auto | Override path to MEMS Audio WAV folder |
| --bruel-npy | auto | Override path to dunakeszi_audit_output/ (.npy caches) |
| --bruel-wav | auto | Override path to raw Brüel WAV folder (pre-flight negatives) |
| --gpx-csv | auto | Override path to gpx_combined.csv |
| --model-dir | auto | Path to drone_v15/models/ |
| --source | both | mems · bruel · both |
| --gt-strategy | all | flight_window · gpx_altitude · gpx_speed · conservative · all |
| --gpx-alt-threshold | 5.0 | Elevation (m AGL) above which segment = drone |
| --gpx-speed-threshold | 1.0 | Speed (m/s) above which segment = drone |
| --segment-sec | 1.0 | Window length in seconds |
| --hop-sec | 0.5 | Hop between windows in seconds |
| --context-min | 10.0 | Non-flight minutes to include from MEMS files each side |
| --mems-start | from filename | UTC datetime when MEMS recording started (ISO format) |
| --bruel-channel | -1 | Channel index (-1 = average all 14 channels) |
| --no-model | off | Skip CNN; heuristic scoring only |
| --cnn-weight | 0.80 | CNN weight in fused score (heuristic gets 1 − w) |
| --threshold | auto | Fixed decision threshold (default: find best F1 on data) |
| --output | ./dunakeszi_benchmark_out | Output directory |

## 4.5  Ground-Truth Strategies

| Strategy | Label = drone when… | Notes |
| --- | --- | --- |
| flight_window | Segment midpoint inside 12:57:21–13:04:09 UTC | Most reliable; no GPX required |
| gpx_altitude | Nearest GPX elevation > --gpx-alt-threshold | Fallback to flight_window if no GPX |
| gpx_speed | Nearest GPX speed > --gpx-speed-threshold | Useful for detecting dynamic flight vs hover |
| conservative | ALL available signals agree | Lowest false-positive rate; most segments excluded |
| all | Runs all four independently | Produces comparison plot across strategies |

## 4.6  Outputs

| File | Description |
| --- | --- |
| mems_all_segments.csv | All MEMS segments with scores + all GT columns |
| bruel_all_segments.csv | All Brüel segments with scores + all GT columns |
| {source}_{strategy}_segments.csv | Per-run filtered segment table |
| {source}_{strategy}_report.json | F1 · AUROC · AUPRC · confusion numbers |
| plots/{source}_{strategy}_timeline.png | Detection probability timeline |
| plots/{source}_{strategy}_roc_pr.png | ROC and Precision-Recall curves |
| plots/{source}_{strategy}_confusion.png | Confusion matrix with percentages |
| plots/{source}_{strategy}_score_dist.png | Score distribution by GT label |
| plots/comparison_table.png | Multi-strategy bar chart (only with --gt-strategy all) |
| benchmark_summary.json | Combined top-line metrics for all runs |

# 5  Session 2 — Vitemor Indoor

## 5.1  Dataset Overview

Three DJI drones were flown sequentially through scripted maneuvers in an indoor studio. 75 event markers annotate every maneuver transition. GPS telemetry uses a local Cartesian coordinate frame (x_rel, y_rel, z_rel in metres) rather than lat/lon.

| Phase | Drone | Recording start (UTC) | Marker range | Key maneuvers |
| --- | --- | --- | --- | --- |
| 1 | DJI Mavic Pro | 11:29:03 | #2 → #19 | Hover 1-4m, square, diagonals |
| 2 | DJI Mavic 2 Pro | 11:49:57 | #20 → #50 | Hover 1-4m, circle, flyover, rotation |
| 3 | DJI Mavic Mini | 11:37:56 | #51 → #66 | Hover 1-4m, square, circle |
| 4 | DJI Mavic 2 Pro | N/A | #67 → #75 | Free run with 7 waypoints (ambiguous GT) |

| Drone | BPF (Hz, median) | GPS clock reliability |
| --- | --- | --- |
| Mavic Pro | 209 Hz | Excellent — T0 aligned to audio ±0 s |
| Mavic 2 Pro | 195 Hz | Degraded — ±40 s spread across 4 hover anchors |
| Mavic Mini | 360 Hz | Degraded — ±20 s spread; pitch/roll axes swapped |

Mavic Mini IMU: pitch and roll columns are swapped relative to the other two drones (~97° pitch, ~3° roll). Account for this before using raw IMU angles for attitude estimation.

## 5.2  Extracting Flight Windows — Step by Step

### Step 1  Locate the Marker File

All 75 markers are in kutatas/markers/250703_Vitemor_Dron_markers_samples.txt. Use sample indices directly — no conversion needed at 192 kHz.

| Time format | File | Best use |
| --- | --- | --- |
| Sample index (192 kHz) | _samples.txt | Programmatic audio slicing — **use this one** |
| MM:SS.mmm | _min-sec.txt | Human-readable quick reference |
| HH:MM:SS:FF at 25 fps | _timecode.txt | Video / DAW synchronisation |

### Step 2  Read the Marker File

import soundfile as sf import numpy as np SR = 192_000 # Marker indices for Mavic Pro hover at 1m and 2m (markers 3 and 4) start_sample = 28_444_702 # marker 3 — "1m lebeg" end_sample = 36_779_185 # marker 4 — "2m lebeg" # Load from one channel of any group-1 file data, _ = sf.read("kutatas/audio_files/Grp1-1_01.wav", start=start_sample, stop=end_sample, dtype="float32", always_2d=True) duration_s = (end_sample - start_sample) / SR print(f"Segment: {duration_s:.1f}s Shape: {data.shape}") # Expected: ~43 s (8 722 s file at 192 kHz = 1.67B samples)

### Step 3  Key Marker Sample Indices

| Event | Sample index | Description |
| --- | --- | --- |
| Sync tap | 1 646 731 | Global sync anchor — align files here if drift suspected |
| Mavic Pro takeoff | 20 449 142 | Phase 1 begins |
| Mavic Pro hover 1m | 28 444 702 | Best static GT segment (Pro) |
| Mavic Pro hover 4m | 56 970 173 | Last static hover (Pro) |
| Mavic Pro landing | 127 360 430 | Phase 1 ends |
| Mavic 2 Pro takeoff | 276 930 583 | Phase 2 begins |
| Mavic 2 Pro hover 1m | 280 711 009 | Best static GT segment (Mv2) |
| Mavic 2 Pro landing | 468 975 616 | Phase 2 ends |
| Mavic Mini takeoff | 663 479 722 | Phase 3 begins |
| Mavic Mini hover 1m | 668 730 776 | Best static GT segment (Mini) |
| Mavic Mini landing | 715 968 682 | Phase 3 ends |
| Free run start | 797 442 048 | Phase 4 — ambiguous GT |

### Step 4  Extract All Hover Segments Programmatically

# Hover event pairs: (start_sample, end_sample, drone, altitude_m) HOVER_EVENTS = [ (28_444_702, 36_779_185, "mavic_pro", 1), (36_779_185, 44_082_717, "mavic_pro", 2), (44_082_717, 56_970_173, "mavic_pro", 3), (56_970_173, 70_669_068, "mavic_pro", 4), (280_711_009, 288_974_716, "mavic_pro2", 1), (288_974_716, 296_137_518, "mavic_pro2", 2), (296_137_518, 301_094_993, "mavic_pro2", 3), (301_094_993, 305_694_795, "mavic_pro2", 4), (668_730_776, 673_925_137, "mavic_mini", 1), (673_925_137, 676_899_179, "mavic_mini", 2), (676_899_179, 678_789_997, "mavic_mini", 3), (678_789_997, 682_124_264, "mavic_mini", 4), ] for s0, s1, drone, alt in HOVER_EVENTS: data, _ = sf.read("kutatas/audio_files/Grp1-1_01.wav", start=s0, stop=s1, dtype="float32", always_2d=True) print(f"{drone:12s} {alt}m {(s1-s0)/SR:.1f}s shape={data.shape}")

## 5.3  Running the Indoor Benchmark

### Quick start (heuristic only, all phases)

python vitemor_indoor_benchmark.py \ --dataset-root ./kutatas \ --no-model

### Hover segments only — best for initial model validation

python vitemor_indoor_benchmark.py \ --dataset-root ./kutatas \ --model-dir ./drone_v15/models \ --maneuver hover

### Mavic 2 Pro only (highest BPF energy ratio — easiest drone to localise)

python vitemor_indoor_benchmark.py \ --dataset-root ./kutatas \ --phase 2

### All phases including free-run, group 2 microphones averaged

python vitemor_indoor_benchmark.py \ --dataset-root ./kutatas \ --group grp2 \ --include-free-run

### Cross-domain comparison (run outdoor benchmark first)

python vitemor_indoor_benchmark.py \ --dataset-root ./kutatas \ --model-dir ./drone_v15/models \ --outdoor-summary ./dunakeszi_benchmark_out/benchmark_summary.json

## 5.4  CLI Reference — Indoor Benchmark

| Flag | Default | Description |
| --- | --- | --- |
| --dataset-root | . | Root folder containing audio_files/, markers/, position/ |
| --audio-dir | auto | Override path to audio_files/ |
| --markers-dir | auto | Override path to markers/ |
| --position-dir | auto | Override path to position/ |
| --q4-csv | auto | Override path to q4_features.csv |
| --model-dir | auto | Path to drone_v15/models/ |
| --outdoor-summary | none | Path to outdoor benchmark_summary.json for cross-domain plot |
| --group | grp1 | grp1 · grp2 · grp3 · all — which mic group to load |
| --channel | -1 | Channel within the group (-1 = average all files in group) |
| --phase | all (1-3) | Restrict to a specific phase (1 · 2 · 3 · 4) |
| --maneuver | all | hover · square · circle · forward · backward · diagonal · flyover · rotation · free_run |
| --include-free-run | off | Include phase 4 segments (labelled ambiguous by default) |
| --segment-sec | 1.0 | Window length in seconds |
| --hop-sec | 0.5 | Hop between windows in seconds |
| --no-model | off | Skip CNN; heuristic scoring only |
| --cnn-weight | 0.80 | CNN weight in fused score |
| --threshold | auto | Fixed decision threshold (default: find best F1 on data) |
| --output | ./vitemor_benchmark_out | Output directory |

## 5.5  Outputs

| File | Description |
| --- | --- |
| indoor_segments.csv | All segments: scores + GT + drone/phase/maneuver/altitude/position |
| indoor_report.json | Overall F1 · AUROC · AUPRC · confusion |
| indoor_per_drone_report.json | Per-drone breakdown (Pro / Mv2 / Mini) |
| indoor_per_maneuver_report.json | Per-maneuver-type breakdown |
| plots/timeline_per_drone.png | Detection timeline with TP/FP/FN coloured by drone |
| plots/roc_pr.png | ROC and Precision-Recall curves |
| plots/confusion.png | Confusion matrix |
| plots/score_dist_by_maneuver.png | Histogram of fused scores grouped by maneuver type |
| plots/bpf_vs_score.png | BPF energy ratio (from q4_features.csv) vs heuristic score scatter |
| plots/cross_domain_comparison.png | Indoor vs outdoor metric comparison (with --outdoor-summary) |
| benchmark_summary.json | Top-line metrics + per-drone + per-maneuver combined |

# 6  Scoring Pipeline — How It Works

## 6.1  Feature Stack

Both benchmarks feed the same feature_stack_v15() function to the CNN. This exactly mirrors what MelCacheManager.build() writes during v15 training — using the wrong features here would silently destroy CNN accuracy.

| Channel | What it is | Normalisation |
| --- | --- | --- |
| 0 — log-mel | Power mel-spectrogram → power_to_db | safe_standardize() per channel |
| 1 — PCEN | Per-Channel Energy Normalisation (librosa.pcen) | safe_standardize() per channel |
| 2 — delta-mel | np.diff of log-mel along time axis | safe_standardize() per channel |

Parameters: n_fft=1024, hop_length=256, n_mels=64, fmin=20, fmax=8000, sr=22050.

## 6.2  Heuristic Scorer

The heuristic is a faithful port of v15's heuristic_detect(). It guards against false positives from indoor HVAC and narrow-band tones via two veto conditions:

- Entropy veto — if median per-frame spectral entropy < 0.38 the signal is narrow-band (tone, siren). Score is capped at 0.25.

- Crest-factor veto — if crest factor > 8.5 AND entropy < 0.52 the signal is impulsive (horn burst). Score is fixed at 0.10.

The indoor benchmark additionally accepts an optional bpf_hz argument. When provided (from q4_features.csv), the band energy component uses ±20 Hz windows around the known BPF harmonics instead of a generic 80–2500 Hz band.

## 6.3  Fused Score

fused = cnn_weight * cnn_prob + (1 - cnn_weight) * heuristic_prob # Bonus +0.06 when both CNN and heuristic independently agree (both > 0.45) if cnn_prob > 0.45 and heuristic_prob > 0.45: fused = min(1.0, fused + 0.06)

Default --cnn-weight 0.80. Set --no-model for heuristic-only scoring (slower to converge but works without a trained checkpoint).

## 6.4  Output Key Names

Both benchmarks expose two sets of identical keys in their segment CSVs for downstream compatibility:

| Benchmark key (CSV) | v15 pipeline key | Meaning |
| --- | --- | --- |
| fused_prob | probability | Primary fused detection score [0, 1] |
| cnn_prob | cnn_probability | Raw CNN softmax output [0, 1] or null if no model |
| heuristic_prob | heuristic_probability | Heuristic-only score [0, 1] |

# 7  Integration with Detection, Localization and Tracking

## 7.1  Detection

After running either benchmark, load the segment CSV and feed the fused_prob column into the v15 classify_detection_score() function:

from drone_detection_v15 import classify_detection_score, config import pandas as pd df = pd.read_csv("dunakeszi_benchmark_out/mems_flight_window_segments.csv") for _, row in df.iterrows(): label = classify_detection_score(row["fused_prob"], config) # Returns: "drone" / "possible_drone" / "weak_possible_drone" / "non_drone"

## 7.2  Localization

For Brüel / indoor segments that have multiple simultaneous channels, pass channel arrays directly to localize() or localize_multi_drone():

# Outdoor — load two Brüel channels from the flight-window cache import numpy as np from scipy.signal import resample_poly from drone_detection_v15 import localize, config arr = np.load("dunakeszi_audit_output/251020VITEMOROM1AT01J_flight_window.npy", mmap_mode="r") SR_BRUEL = 192_000; SR_MODEL = 22_050 start_frame = 0; end_frame = int(1.0 * SR_BRUEL) # 1-second window channels = [] for ch in range(3): # use first 3 channels as mic array y = arr[start_frame:end_frame, ch].astype("float32") y_rs = resample_poly(y, SR_MODEL, SR_BRUEL).astype("float32") channels.append(y_rs) loc = localize(channels, config) print(f"Azimuth: {loc['azimuth_deg']:.1f}° Distance: {loc['distance_m']:.2f} m")

## 7.3  Path Tracking

Wrap the localization output in the KalmanTracker from v15 to build continuous tracks:

from drone_detection_v15 import KalmanTracker, config import numpy as np tracker = KalmanTracker(config) # Iterate over 1-second windows in the flight-window cache arr = np.load("dunakeszi_audit_output/251020VITEMOROM1AT01J_flight_window.npy", mmap_mode="r") SR = 192_000; WIN = SR # 1-second windows for i in range(len(arr) // WIN): window = arr[i*WIN:(i+1)*WIN, :3] # channels 0-2 channels = [resample_poly(window[:, c], 22050, SR) for c in range(3)] det = detect(channels, config) if det["detected"]: loc = localize(channels, config) confirmed = tracker.step([loc["xy_position"]], timestamp=i) print(f"t={i}s tracks={len(confirmed)}") # Save all confirmed tracks tracker.save(Path("tracks_dunakeszi.json"))

## 7.4  Indoor — Using Marker Segments as Direct Inputs

The indoor dataset's static hover segments are ideal for a controlled localization accuracy test because the drone position is known from telemetry:

# Load one hover segment and run the full pipeline import soundfile as sf from scipy.signal import resample_poly from drone_detection_v15 import detect, localize, config SR_INDOOR = 192_000; SR_MODEL = 22_050 # Mavic 2 Pro hover at 3m (markers 23-24) S0, S1 = 296_137_518, 301_094_993 # Load 3 microphones from group 1 channels = [] for mic in range(1, 4): data, _ = sf.read(f"kutatas/audio_files/Grp1-{mic}_01.wav", start=S0, stop=S1, dtype="float32", always_2d=True) mono = data[:, 0] # or average across columns channels.append(resample_poly(mono, SR_MODEL, SR_INDOOR).astype("float32")) det = detect(channels, config) print(f"Detected: {det['detected']} prob={det['probability']:.3f}") if det["detected"]: loc = localize(channels, config) # Ground truth: x_rel≈0, y_rel≈0, z_rel≈3.0 from mavic_pro2.csv print(f"Az={loc['azimuth_deg']:.1f}° Dist={loc['distance_m']:.2f}m")

# 8  Training the Model on These Datasets

## 8.1  Using the Dataset Builder

Clean drone clips from either session can be fed directly into custom_drone_dataset_builder.py to create augmented training data:

# Step 1 — extract clean segments from a session python custom_drone_dataset_builder.py \ --source ./kutatas/audio_files \ --backgrounds ./backgrounds \ --output ./dataset_out # Step 2 — train with the new dataset from drone_detection_v15 import train_detection, quickstart_notebook_setup, config quickstart_notebook_setup( config, dataset_root="./dataset_out", include_background_pool_as_non_drone=True, ) train_detection( config, epochs=5, resume=False, force_rebuild_cache=True, use_custom_builder=True, )

## 8.2  BPF Configuration

When training on indoor data, update the DRONE_BPF_HZ constant in dunakeszi_audit_v2.py and the BPF_HZ dict in vitemor_indoor_benchmark.py to match the session-specific values from q4_features.csv:

| Drone | Median BPF (Hz) | Expected band (±20 Hz) | Note |
| --- | --- | --- | --- |
| Mavic Pro | 209 | 189 – 229 Hz | 2m anomaly: BPF shifted to 279 Hz in one segment |
| Mavic 2 Pro | 195 | 175 – 215 Hz | Most consistent across altitudes (193–199 Hz) |
| Mavic Mini | 360 | 340 – 380 Hz | Highest BPF; well-separated from indoor noise peaks |

# 9  Known Issues and Caveats

## 9.1  Outdoor Session

- `--mems-start` required — MEMS filenames do not contain a timestamp. If the default 2025-10-20T10:00:00Z does not match your recording start time, pass the correct value or the GT labels on MEMS segments will be wrong.

- Brüel `.npy` caches must exist — the benchmark reads pre-extracted caches from dunakeszi_audit_v2.py. If they are missing, run the audit script first or point --bruel-npy at the output folder.

- GPX clock drift — the outdoor GPX data has ~80–90 ms jitter at ~9 Hz. The gpx_altitude and gpx_speed strategies are therefore approximate; flight_window is the authoritative GT.

- `group3_audio` (indoor) — the 4 auxiliary files are ~6× shorter than the main 12-channel files. Their channel-to-microphone mapping and spatial positions are not yet documented. Treat with caution.

## 9.2  Indoor Session

- HVAC interference at 627 Hz — coincides exactly with the 3rd harmonic of the Mavic Pro BPF (3 × 209 Hz). The entropy veto suppresses this in silence windows, but it may slightly inflate heuristic scores for the Mavic Pro during non-hover segments.

- Phase 4 free-run excluded by default — segments from markers 67–75 are labelled -1 (ambiguous) and filtered out of all metric calculations. Pass --include-free-run to include them as drone=1.

- Mavic 2 Pro and Mini GPS unreliable — T0 alignment spread is ±20–40 s. Only use z_rel (altitude) for ground truth in hover segments where the position is approximately constant. Do not use for dynamic segment positioning.

- Pro 2m anomaly — BPF at Mavic Pro 2m hover shifted to 279 Hz (vs 209 Hz at all other altitudes). BPF energy ratio is 0.098 for that segment — anomalously low. The cause is unclear (RPM change or rotor anomaly). The segment is still labelled correctly as drone=1.

## 9.3  Compatibility

- Both benchmark scripts share the identical scoring stack (feature_stack_v15, heuristic_score, cnn_score, score_segment). If you update the heuristic weights or feature parameters in one script, update the other.

- The CNN model was trained at SR=22050 with [mel, PCEN, delta-mel] features. Feeding [mel, mel, mel] (as the original benchmark v1 did) causes silent accuracy degradation.

- soundfile is required for the indoor benchmark (marker-based seek loading). scipy.io.wavfile loads whole files and cannot seek efficiently into 2.9 GB WAVs.

# 10  Quick Reference — End-to-End Workflow

# ── 1. Pre-process outdoor audio (one-time) ────────────────────────────── python dunakeszi_audit_v2.py # ── 2. Train the model ─────────────────────────────────────────────────── # (in drone_detection_v15.py or a notebook) train_all(config, det_epochs=10, loc_epochs=10, resume=False) # ── 3. Run outdoor benchmark ───────────────────────────────────────────── python dunakeszi_benchmark.py \ --dataset-root ./dunakeszi_data \ --model-dir ./drone_v15/models \ --source both --gt-strategy all # ── 4. Run indoor benchmark ────────────────────────────────────────────── python vitemor_indoor_benchmark.py \ --dataset-root ./kutatas \ --model-dir ./drone_v15/models \ --outdoor-summary ./dunakeszi_benchmark_out/benchmark_summary.json # ── 5. Inspect results ─────────────────────────────────────────────────── cat dunakeszi_benchmark_out/benchmark_summary.json cat vitemor_benchmark_out/benchmark_summary.json # → Cross-domain comparison plot: vitemor_benchmark_out/plots/cross_domain_comparison.png